# Peek and poke tutorial

By RubyNL

If you have looked up some source and wondered why you didn’t found one single PSET in it, or just wondered how to the fast, flicker-free graphics that you see sometimes, for example in demo’s, this tutorial is the right thing to you. I’ll teach you about peek and poke, which can be used as an alternative for point and pset, and some other things.

## What are peek and poke?

Peek and poke are two instructions that you use to look up bytes at certain positions. With poke you can set a byte at a certain position a certain value. With peek you look up a byte at a certain address. So, for the clearness:

<code>

POKE position, value

value = PEEK(position)

<.code>

In case you don’t know, a byte is not quite the same as an integer. While an integer contains a value between -32768 and 32767, a byte is a value between 0 and 255. So, these bytes are perfect to represent a colour at some point in the screen. First, I’ll teach you how to use peek and poke for normal variables. Before that, I have some theory about memory for you. That means, no code yet. Sorry.

## Memory theory

Something you need to know about is memory segments. A segment of memory is a piece out of the memory. Its size is 65535 bytes(= 32767 integers). You can only peek and poke in the current segment. The segment can be anywhere in the memory, as long as it starts somewhere and ends 65535 bytes further. The default segment is starts at 0, so it stops at 65536. When you need much memory(more then 32767 integers) you need to change the segment.

## Actually using peek and poke

Now, come on with the code! Before you read the example proggie, I will explain the some things:

VARPTR is used address the integer that follows. A variable that is used to address (point) to an integer, is called a pointer. Instead of address you can use VARPTR(variable), but when you need to have the address to the variable more frequently, it is faster to save it in a variable.

<code>

CLS

DIM address AS INTEGER

DIM variable AS INTEGER

address = VARPTR(variable)

'let address be the pointer to the integer variable

variable = 10

PRINT variable

PRINT PEEK(address)

POKE address, 15

PRINT variable

PRINT PEEK(address)

</code>

When you look at this code, you can see that poking to address is the same as setting variable to another value. But, this only is like that if variable is a value under 255. Why? Well, remember that one byte can only have a value from 0-255? And with peek, you only read one byte. So, cuz one integer is two bytes, we need to peek another byte. Which? Simply the next one. And, for integers, we use this formula:

<code>

Value = PEEK(address) + PEEK(address + 1) * 256

</code>

Value is the value of the variable where address points to. This formula is only true for positive values of the variable.

For negative values of variable, we subtract 256 from the second bit:

<code>

Value = PEEK(address) + (PEEK(address + 1) – 256) * 256

</code>

### Poking to the screen

Note: The ideas and code I present here, are only for SCREEN 13! Be sure that you’re in screen 13 when you use this code!

Now you know how to look up the values if integers with peek and poke. But actually, this is quite useless and I just cover this for completion and for learning purposes. It is only to give you the idea of how the memory works. To help you with a really useful trick you can do with peek and poke, you have to know how the screen is saved in the memory:

The screen (in screen 13) has 320 * 200 = 64000 pixels. Each pixel can have a value from 0-255. Hey! This exactly the same value a byte can have. And, it won’t surprise you: each pixel is one byte in memory. But how you know which coordinates (x: 0-319, y: 0-199) are with which offset(0-63999)? When you have the coordinates and you want to get the offset, use this formula:

<code>

offset& = y * 320& + x

</code>

Then you poke or peek at that offset. Poke to set the pixel at that offset to a certain colour, and peeking to get the colour at that offset. Like this you can use peeking and poking instead of psetting and pointing. When you poke a value or colour more then 255, the counting will start over again. So 256 becomes 0, 257 becomes 1, just like -1 becomes 255, -2 becomes 254…

Be sure that you use LONG integers for the offset, and a LONG sign after the 320, to avoid an overflow error. But, of course you can’t peek and poke to that yet, because the screen offset doesn’t start at the starting segment of qbasic. It actually is at &HA000. That looks weird, because it is a hex number(hex numbers have &H before it always in qbasic). We use hex numbers just because &HA000 is easier to remember then -24576(the matching decimal integer for the hex number &HA000). Now you set the segment to &HA000 or -24576, you can poke to it where you want. Code:

<code>

DEF SEG = &HA000

</code>

When you’re ready with poking to the memory, always return to the normal segment by DEF SEG (without any arguments). When you don’t do this there may be some weird errors (screens that freeze and a bunch of other things).

That is a useful feature of peeking and poking: it is much faster then pset and point.

But this can still be made a lot faster. We can either replace that multiply with a lookup table with a size of 199 that multiplies every value you pass to it with 320. Make sure that you multiply with 320& and use a long integer-type array. An other way to do it fast, and avoid that some memory is wasted by a lookup table, is just to access the screen in the way it is stored in memory: row by row. You just initialise the pointer to zero, and after you plotted a pixel, you add one to the pointer to go the next pixel. Be sure that the x loop is the inner loop and the y the outer loop! Else it will mess up. Here’s some code.

<code>

SCREEN 13

DEFINT A-Z

DEF SEG = &HA000

pointer& = 0

FOR y = 0 TO 199

FOR x = 0 TO 319

POKE pointer&, colour

pointer& = pointer& + 1

NEXT

NEXT

DEF SEG

</code>

Of course, the pointer is a long integer again.

There are still some faster and faster ways to get an image to the screen. That is with a buffer. BTW, I still want to teach you something extras about the VARSEG shit.

VARSEG and VARPTR

Do you remember the value of an integer, that was stored in memory with the second bit representing a value 0-255 that needs to be multiplied with 256? Well, varseg and varptr work actually the same way. Only, varseg is 16 times the pointer instead of 256(that is because the segment is shifted 4 bits to the left, if you understand, and care). Instead of adding 16 to the pointer, you could add one to the segment. The same logic: instead of adding 320 to the pointer, we could add 20 to the segment. So, instead of drawing the whole screen with one long integer, we can draw the screen with one integer, and update the segment every line. We do this like this:

<code>

DEF SEG = &HA000 + y * 20

</code>

Then we draw like this:

<code>

FOR x = 0 TO 319

POKE x, colour

NEXT

</code>

This is not really faster then the other ways I showed you, but it is added for completion, and to show you how varseg and varptr relate to each other. BTW, some programs use this, and now you know exactly what’s happening ;). It is possible to get the multiply by 20 out, and then it is, as I believes, the fastest way of poking to the screen. You just add a variable that increases with 20 each time on the end of the y-loop. You don’t need a long integer anymore that makes it slow. The code is:

<code>

SCREEN 13

DEFINT A-Z

yp = 20

FOR y = 0 TO 199

DEF SEG = &HA000 + yp

FOR x = 0 TO 319

POKE x, colour

NEXT

yp = yp + 20

NEXT

</code>

Of course, you can replace the &HA000 segment by a buffer segment(you will learn later how to do this). The fastest way, however, would be to write some ASM routines. One to write to the buffer, some kind of PSET, and another to copy the buffer to &HA000. This is not that hard to do, and well worth the effort when you’re, for instance, making a game. It is also a fairly good practice for your ASM coding.

### Poking to a buffer

When you want to have flicker free graphics, a buffer is very useful. It eliminates almost every flickering. The trick with a buffer is to set the segment not to &HA000, but to the segment of a buffer. The actual buffer is just a large array of short integers. The buffer needs to have enough memory for

- All pixel colours

- The height and width information

The size for the pixels is easily calculated by just multiplying the width of the pic by the height of it. Then you divide by two, because we don’t use bytes but integers, which exist out of two bytes. Then you add two, because the information needed to put the pic to the screen is two integers. Formula:

Size = INT(width * height / 2 + .5) + 2

So, for a screen size buffer you get (320 * 200) / 2 + 2 = 32002 elements. So, you dim an integer array with 32002 elements:

<code>

DIM buffer(32001) AS INTEGER

</code>

That actually are 32002 integers, because the counting starts at 0. So 0-32001 elements are 32002 elements ;).

Then you set the segment to the segment of the buffer:

<code>

DEF SEG = VARSEG(buffer(0))

</code>

Or, when you want to use this segment more frequently, you can first save the segment in an integer:

<code>

buffersegment = VARSEG(buffer(0))

DEF SEG = buffersegment

</code>

But before you try this, you have to give up the width and height of the buffer too. This is called initialising the buffer. It is needed to blit the buffer to the screen. We do it in the following way: we set the first element(in this example: buffer(0)) to 8 times the width. I don’t exactly know why the width needs to be multiplied by eight, but be sure to do it, else it won’t work. The second element is set to the height. In code:

<code>

buffer(0) = 8 * bufferwidth

buffer(1) = bufferheight

</code>

Ok, now you’ve set the segment and initialise the buffer, you’re ready to poke to it. One thing I have to tell you about: there are easier ways to use a buffer. With get and put. Once you have dimmed an array with enough space you can get any portion of the screen with this command:

<code>

GET (x1, y1) – (x2, y2), buffer(0)

</code>

And put it on any position:

<code>

PUT (x, y), buffer(0), PSET

</code>

With PSET you make sure that the buffer overwrites the background. When you don’t use it, it looks very weird. Not recommended :P.

An easier way to initialise the buffer is to simply GET the whole screen:

<code>

GET (0, 0) – (319, 199), buffer(0)

</code>

But, this copies the whole screen into your buffer, so when you want to have a empty buffer your screen needs to be empty too ;). I told you about the height/width manner too, because you need to know how GET/PUT work. GET initialises the array automatically with the width and height, and PUT blits the buffer super-fast to the screen.

You’re almost ready to use the buffer, the only thing to know is that your pointer starts with a value(offset) of 4. This is because the first two integers are used to save the height and width. And each integer is two bytes. So, when we put everything together:

<code>

SCREEN 13

DEFINT A-Z

DIM buffer(32001)

buffer(0) = 320 * 8

buffer(1) = 200

pointer& = 4

FOR y = 0 TO 199

FOR x = 0 TO 319

POKE pointer&, RND * 255

pointer& = pointer& + 1

NEXT

NEXT
PUT (0, 0), buffer(0), PSET

</code>

Well, it’s not that hard, huh? And, the nicest, it gets rid of all flicker! When your having memory problems, try using REM $DYNAMIC in your code. It gives you more memory to use. Very handy when you use a buffer, because a buffer eats up almost all memory. The only little downside of a buffer. But when you use dynamic memory, with REM $DYNAMIC, you have no problems with that. One last thing, when you have still some flicker, try this:

<code>

WAIT &H3DA, 8

WAIT &H3DA, 8, 8

</code>

Right before you PUT the buffer to the screen. That should eliminate all flicker. Bang :P.

## Quick filtering

I will present a way to do very fast blurring or a fast fire filter. You can do this by poke at the current pixel and peek at the right places: for a blur filter, peek at the surrounding pixels, so:

Pixel above: PEEK(p& - 320)

Pixel left: PEEK(p& - 1)

Pixel right: PEEK(p& + 1)

Pixel down: PEEK(p& + 320)

Then you divide the value of all this pixels together, and poke this at the pointer. You may need a loop from x: 1 to 318 and y: 1 to 198. This is a bit weird coded, because the pointer needs to skip the first and last pixels of a line. Also notice that you need to have a gradient palette to blur. Code:

<code>

SCREEN 13

DEFINT A-Z

FOR c = 0 TO 255

OUT &H3C8, c

OUT &H3C9, c / 255 * 63

OUT &H3C9, c / 255 * 63

OUT &H3C9, c / 255 * 63

NEXT

DEF SEG = &HA000

P& = 0

FOR y = 0 TO 199

FOR x = 0 TO 319

p& = p& + 1

POKE p&, RND * 255

NEXT

NEXT

DO UNTIL LEN(INKEY$)

p& = 321

FOR y = 1 TO 198

FOR x = 1 TO 318

total = PEEK(p& - 320) + PEEK(p& - 1) + PEEK(p& + 1) + PEEK(p& + 320)

POKE p&, total \ 4

p& = p& + 1

NEXT

p& = p& + 2

NEXT

LOOP

DEF SEG

</code>

You can make a LUT for the divide by four. The size of the LUT needs to be from 0 to 4 * 255 = 1020. You can add the centre pixel too and divide by five. The size needs to be 5 * 255 = 1275 then. This gives a slightly less, but better blur.

If you want to copy one buffer to another, you just make a 0-199 loop and swap the segment every time in the inner loop, like I told you in ‘VARSEG and VARPTR’. Between the PEEKing from the one buffer and POKEing to the other, you need to save the values in a LUT, which can hold 319 integers.

If you want to know more:

Memory tutorial (© by Qbasic for All!) – Teaches you all about the memory in Qbasic(not about using it or implemention, but just how all the commands concerning memory):

http://www.petesqbsite.com/sections/tutorials/tuts/memory.htm

Get and put tutorial by Andrew L. Ayers – All about GET and PUT.

http://www.petesqbsite.com/sections/tutorials/tutorials/getput13.txt

## Extra

I actually finished this tutorial, but then I though about something that Galleon learned me. Credits go to him.

It is another way of using a buffer, but now you write to it with Qbasic’s graphical functions(PSET, POINT, LINE, CIRCLE, GET, PUT). I think that it is very cool when you understand it. But, it’s gonna be a bit complicated, so don’t worry when you don’t understand. That’s why I called this part “Extra”.

What I’m going to try to explain to you, is a way to use Qbasic’s graphical functions with a buffer. So, when you use LINE, it draws a line to the buffer. Unfortunately, this doesn’t work for PRINT because text uses another segment.

First, we have this idea:

We know that Qbasic build-in graphical functions writes pixels to the screen. But we want qbasic to write to our buffer. I accidentally know that the place where Qbasic lets its graphical functions draw, is stored somewhere in memory. So somewhere in the memory must be standing: &HA000. OK. Now we must translate this idea to code:

Search in the whole default segment of Qbasic (0-65535) for &HA000. When you make &HA000 an integer, and you look up each byte, you get:

First byte: 0

Second byte: 160

So it is easier to make an IF into the loop:

<code>

'Scan the whole memory

FOR pointer& = 0 TO 65534

IF PEEK(pointer&) = 0 AND PEEK(pointer& + 1) = 160 THEN

'You’ve found &HA000, now check if it’s the right place in the memory

</code>

Now we need to check if it’s actually the right place in the memory. We do this by messing up the place where &HA000 stood first. Then we PSET a colour at (0, 0). Then we peek a value at segment &HA000, offset 0. If that is the colour, then it is not the right place in memory, so we restore &HA000 at the current offset and we need to continue searching. If it is not that colour, we save the current value of pointer&, and stop searching:

<code>

'We mess up the first byte(we make it 1 instead of 0)

POKE pointer&, 1

'Then we try if PSET still PSETs at &HA000

PSET (0, 0), 127

'If it does not, we have the right place

DEF SEG = &HA000

IF PEEK(0) = 127 THEN

'We have a wrong place, restore pixel at (0,0), and get &HA000 back into memory

DEF SEG

PSET (0, 0), 0

POKE pointer&, 0

ELSE

'We found the right offset, so get the buffer segment into the place of &HA000, then exit from the FOR loop

DEF SEG

place& = pointer&

</code>

Now we need to get the segment of the buffer into the memory. But here is a little problem, because you can only set a segment in the memory and no pointer. But we want to start the drawing at buffer(2), because the first two elements are saved for the height and width of the pic. We can’t increase the segment by two due to the fact that the segment is 16 times the offset. So an offset of 1(byte) in the segment would mean 16 bytes in the pointer. Hey! Now we can use it! We just dim our buffer a bit bigger then normal, and let the drawing start at buffer(8), which is the 16th byte in the array. So the width is set at buffer(6) and the height at buffer(7). Then we can PUT it to the screen with:

<code>

PUT (0, 0), buffer(6), PSET

</code>

Also, you can easily initialise it with GET again:

<code>

GET (0, 0) – (319, 199), buffer(6)

</code>

Now we need to write the segment(VARSEG(buffer(8))) into memory. The memory is split up in bytes, we need to split up the segment in bytes. We do this by letting the first byte be the value AND 255, and the second byte the value \ 256. Code:

<code>

Segment = VARSEG(buffer(0)) + 1

POKE pointer&, segment AND 255

POKE pointer& + 1, segment \ 256

</code>

This manner(ANDing by 255 for the first byte and integer dividing by 256 for the second) works also fine with writing positive values to integers.

Now you’ve done all this, we’re almost finished. One important thing to know is that you can’t PUT to the screen no more, because you PUT to the buffer. When you want to put to the screen, you will need to restore the &HA000 segment in memory. Therefore, you need to save the pointer into a variable. To restore the &HA000 segment:

<code>

POKE place&, 0

POKE place& + 1, 160

</code>

Then you put the pic to the screen and then you restore the segment of the buffer in memory.

It’s recommended to put this into two subs, one for initialising the buffer and finding the &HA000 in the memory, and the other to put the buffer to the screen. Well, I will give you the whole code with some code to test it:

Note: the SUB initialises the buffer, searches the memory for &HA000 and sets the buffer segment in the place of &HA000.

The SUB refresh sets the segment in memory back to &HA000, blits the buffer to &HA000, then sets the segment back.

<code>

DECLARE SUB refresh ()

DECLARE SUB initialise ()

CLS

SCREEN 13

DEFINT A-Z

DIM SHARED buffer(32007)

DIM SHARED segment, firstbyte, secondbyte

DIM SHARED place AS LONG

initialise

DO

'Use this instead of CLS:

LINE (0, 0)-(319, 199), 0, BF

'(CLS will work on &HA000 always, and we only want to clean the buffer)

FOR x& = 0 TO 10000

PSET (RND * 319, RND * 199), RND * 255

NEXT

refresh

LOOP

SUB initialise

'Initialise the buffer, set the segment and the two bytes

buffer(6) = 320 * 8

buffer(7) = 200

segment = VARSEG(buffer(0)) + 1

firstbyte = segment AND 255

secondbyte = segment \ 256

FOR pointer& = 0 TO 65534

IF PEEK(pointer&) = 0 AND PEEK(pointer& + 1) = 160 THEN

'You’ve found &HA000, now check if it’s the right place in the memory

'We mess up the first byte(we make it 1 instead of 0)

POKE pointer&, 1

'Then we try if PSET still PSETs at &HA000

PSET (0, 0), 127

'If it does not, we have the right place

DEF SEG = &HA000

IF PEEK(0) = 127 THEN

'We have a wrong place, restore pixel at (0,0), and get &HA000 back into memory

'then continue with the loop

DEF SEG

PSET (0, 0), 0

POKE pointer&, 0

ELSE

'We found the right offset, so get the buffer segment into the place of &HA000

'and save pointer& into place&, then exit from FOR loop

DEF SEG

place& = pointer&

POKE place&, firstbyte

POKE place& + 1, secondbyte

EXIT FOR

END IF

END IF

NEXT

END SUB

SUB refresh

'Set memory to &HA000

POKE place&, 0

POKE place& + 1, 160

'Put the buffer to &HA000

PUT (0, 0), buffer(6), PSET

'Restore buffer segment into memory

POKE place&, firstbyte

POKE place& + 1, secondbyte

END SUB

</code>

## Conclusion

I hope you liked this tut, and everything was clear. You may send me some feedback on

basicallybest@live.nl. BTW, thanks for reading the tut.

And also don’t bother to mail me if you don’t understand something. Also check out my site:

http://members.lycos.nl/rubynl

It’s a bit old, but I’m doing a re-organization soon.

Greets,

Codemss/RubyNL (Codemss is my new username).

## Credits

Plasma for SetVideoSeg